# Real-time RPE Prototype ICD

Version 5.8
Michael Schwartz, MIT Lincoln Laboratories, August 4, 2017

Table of Contents

Revisions	2

References	3

Open Issues	3

Introduction	3

Types of information transferred	4

Data Channels	4

Network protocol for online operation	5

Parameter Inputs for radar data	5

Radar Data Streams	6

Basic Workflow	6

Offline Reach-back for the C array	7

System Status	7

Http Error Codes	9

Requests for Streaming RDM data	9

Protocol	9

Request to Process a channel – Start	10

Request to Process a channel – Stop	11

Request to Initiate Radar Data Transfer – Start	11

Header Data	12

Request for Radar Data	17

RDM Radar Data	18

Request to Initiate Transfer - Halt	18

Offline RPEI Processing	19

Sounder Data requests	20

Sounder scheduling	21

Online Sounder Processing	21

Initiate Sounder Data Transfer	22

Transfer sounder data	23

Halt Sounder Data Transfer	24

Offline Sounder Processing	24

Request Clutter Map	25

Unimplemented features	26

Sync Check	27

Appendix Google Protobuf Definitions	28

Status return	28

Start Transfer Request	29

RDM Header	30

ClutterMap request	33

Sounder Request	33

Sounder Response	33

# Revisions

- 5.0:

  - encode RDM as amplitude/azimuth

  - use BCC record as part of request for read data

  - add wide beam rdm; offline RDM; clutter map and noise map; sounder data

- 5.1:

  - Remove design section

  - Encode RDM as SFFT -compatible complex interference values

  - Azimuth specification for OI sounder

  - Use legacy encoding of Radar data

  - Remove https

- 5.2

  - Add missing BCC fields

  - Add additional background RDM files

  - Eliminate vi return, revise sounder header  to look more line Exmore header

- 5.3

  - Revise offline rdm file creation

- 5.4

  - Wide and narrow beam rdm files are streamed

  - Modify data transfer requests to support multiple streams from the same channel

  - Change default wide beam azimuth

  - Modify oblique sounder default azimuths

- 5.5

  - Radar data format consistent with final agreement

  - Specify SLI for offline Radar

  - Offline sounder

  - Modify sounder return

- 5.6

  - Mention need for client to support http redirect

  - Update to SFFT v7 and ROP header version 6

  - Add flags for RDM normalization and SAP

  - Clarify changing azimuth in online

  - Update sounder header and add Protobuf definitions

- 5.7

  - Add site convention for raw file directory labeling

  - Discussion of how online sounder relates to scheduling

  - Modify Clutter Map interface

  - Emphsize need for ground range table

- 5.8

  - Reachback for Charlie for offline processing

  - ROP version 7

# References

- CDRL A001 “BCC / Cap Demo Computer Interface Control Document”,  General Dynamics Mission Systems, 15 June 2015

# Open Issues

# Introduction

This document describes the real-time RPE prototype software.  The purpose of this software is to make RPE data available to the Scimitar and Exmore systems.  These two systems are assumed to be installed on two different computer systems – a system running the Exmore and Scimitar software and a second system running Lincoln Laboratory’s real-time software connected to the RPE receiver interfaces.

This document describes two versions of the interface: a streaming interface used for real-time processing and a file-based interface used for offline processing.  The streaming interface defines a REST API which the Exmore client can use to ask for data and also ask for information about the status of the system.

# Types of information transferred

The interface transfers several types of data from the RPE system to the Exmore/Scimitar system.  The formats for each data type and the request format for each data type is described in later parts of this document.

- Status – This data describes the condition of the processing chain and of the data streams passing from the processing chain to the receiver

- Radar Data – This data shows the radar returns. The data contains a header giving operational parameters followed by amplitude and azimuth data for the range and Doppler bins specified by the input processing parameters.  This data will be available in three data streams:

  - Streamed data whose azimuth and beamwidth are specified by the operator

  - Streamed wide-beam data.

  - Streamed narrow-beam data.

The last two streams will be available at a different CIT and SLI than the first stream.

- Sounder data – This is used for coordinate registration. The RPEI interface will provide oblique sounder (wsbi) data, but will not provide quasi-vertical or forward oblique data.  Vertical sounder data will continue to be available to Exmore using the existing BAE SRS system. The backscatter oblique sounder data consists of time delay as a function of frequency and as a function of angle. This data will be available in two forms:

  - A data stream – in this form the Exmore client will get data from the current sounder.

  - An offline form. The user will be able to run a program which processes a specified sounder file to produce a file which can be imported into Scimitar.

- Clutter map – This data shows the ground return as a function of azimuth and range for a specified time. It will be in a form which can be uploaded into NASA World Wind. This data is available as a streamed kmz file.

# Data Channels

The Cap Demo receivers produce three or more channels of data, each of which can have its own waveform. In normal operation, the third channel is used for sounder data. In the current CD3 real-time system, there is a separate computer dedicated to each of the radar channels. Because the RPEI interface will be making use of the current CD3 real-time processing chain, and because of the resource demands from the processing chain the RPEI interface will be able to support the two radar channels concurrently only if it is set up on two separate computers.

The client API will contain information about the antenna and channel number.  Calls which need to specify the antenna and channel will use the letters A, B … to designate the antenna and the numbers 1,2 ,… to designate the channel.

# Network protocol for online operation

The RPEI system will use an http-based REST interface for transferring data streams to the Exmore client. This means that Exmore will only receive data it asks for when it asks for it. Because most of the RPEI processing operations are resource-intensive, the RPEI implementation will use more than one server.

# Parameter Inputs for radar data

Because the legacy system does not digitize the input to each receiver, it is necessary for the operator the specify parameters related to the angle being observed. In addition, the RPE processing software needs to have parameters which control Doppler processing. Because the Exmore system will also be able to ask for data from the recent past, the legacy system must also be able to specify a starting time for the data. For these reasons RPEI processing require the following additional parameters

- Azimuth

- Receive Beamwidth

- First and last azimuth used for the wide beam return

- Elevation

- Center Range (used by Scimitar as the default display center)

- Sweeps per CPI

- CPI overlap percentage (1-SLI/Coherent Integration Time)

- Start time relative to present

Note that ARI is not present. For the max beam algorithm, the value of the ARI is arbitrary, so for the RPEI interface it will be set using the following procedure:

- Set the initial ARI to twice the beam width (ari factor = 2)

- Compute an effective baseline separation corresponding to this ARI

- Adjust the baseline so it is an integral multiple of the distance between receiver elements

- Compute a final ARI based on the adjusted baseline.

There are additional parameters which determine whether the RPEI signal chain will include two different types of enhanced processing:

- SAP (Spatial Adaptive Processing) – applied to the main processing chain

- RDM normalization – applied to the final RDM computation

Each of these operations will be applied only if the client explicitly asks for them.

In addition to processing parameters the Exmore system will also communicate waveform parameters to the RPEI system. It will do this using the BCC parameters defined reference 1.

# Radar Data Streams

The RPEI system produces three data streams from a given data channel:

- A stream for the Exmore operator. This stream is displayed on the Exmore system. The azimuth, beamwidth, elevation, sweeps per cpi, and SLI for this stream are specified by the operator.

- Two streams for use during post-processing. Exmore will not display these streams but will instead save them as part of the CDS for this mission. Each of these streams will have an azimuth equal to the center azimuth of the transmitter beam. The beamwidths for these two streams will be specified by a configuration file. The initial values will be 10° and 30°, giving narrow and wide beam files. The wide-beam and narrow-beam streams will share common values for the sweeps per CPI and SLI. These values will be specified by a configuration file.  The initial values are 128 sweeps per CPI and 1.00 seconds for the SLI.

# Basic Workflow

Two basic workflows are possible. In the real-time workflow the Cap Demo system streams data to the Exmore system. In the offline workflow the Cap demo produces files which are read in by the Scimitar system for post-processing.

For the real-time workflow, the operator of the legacy system will ask for RPEI data. This request will consist of several parts:

- Exmore will ask the RPEI system to process a specified channel for a specified receiver array. The process channel request will contain

  - The BCC parameters to use

  - The sweeps per CPI and SLI values to use for the operator stream (the values for the wide and narrow beam streams are specified by configuration values)

  - Flags specifying whether to use Spatial adaptive processing and RDM normalization

- For each of the data streams, Exmore will make a request to initiate transfer. This request will contain

  - The type of stream (operator stream, wide-beam stream, narrow-beam stream).

  - In the case of a request to start the operator stream this request will also contain the desired azimuth, beamwidth, and elevation.

  - In the case of a request to start the operator stream, the start time for the stream, relative to the present.

The return will contain the header information for the requested stream.

- For each of the data streams, one or more requests for more data. The return will contain

  - The number of CPIs being returned. In the case where the real-time operator has changed parameters, this value will be -1

  - The radar values

For each separate stream, Exmore has the option of making the requests for that stream on a separate thread.

Exmore has the option of stopping any individual stream at any time and then resuming it. In the case of the operator stream, this means that Exmore has the option of taking a second look at the data at a different azimuth or beamwidth. In the case of the operator stream this second look at the data can start at a specified number of minutes in the past. If all three streams are stopped, processing for the data will stop. Exmore can also halt processing of the channel, in which case all three streams will stop. Data processing will also stop if any of the waveform parameters change or if the transmitter changes.

For offline processing, the user will run an application running on the RPEI system. That application will allow the operator to specify inputs specific to the legacy system and will create a disk file, which the user can transfer. Once the file is transferred, the operator must run a second program which will update the data base appropriately. It will also be possible for this second program to invoke the RPEI program. In this way an operator on the Scimitar workstation can ask for an RPEI file directly.

## Offline Reach-back for the C array

The offline processing capability is designed to work from a set of raw files stored on a file server. However, at the current operational site for RPEI the support servers for the C array are set up in such a way that the RPEI offline server has access to raw files which have only recently been stored and which have not yet been transferred to the archival NAS. There files are distributed by receiver number among 6 “BE” servers. In order to allow the offline RPEI server to process these data files the site configuration has been modified as follows:

- The data directories from each of the C array BE servers have been mounted on the offline server. Their mount points are at /data/charlie/be01, /data/charlie/be02, …

- The offline processing software has been modified to recognize raw file path names of the form /data/Charlie/be01/etc and process data from paths of that form appropriately.

Thus in order to use the reach-back feature for sounders or for radar it is only necessary to use the correct type of base path for the raw file. A conventional base path will be processed normally.

# System Status

The RPEI system will support a status request which will give the Exmore operator some idea of the state of the Cap Demo system. Because this is a proof-of-concept prototype, the information returned will not be complete, but will allow the Exmore client to see if the system is up and functioning. The information returned by the status request will contain the following information:

- Whether the signal processing chain is up and running

- Whether the system is currently transferring data

- Which receivers are actively transferring data

- If the system is transferring data, the following waveform parameters

  - Frequency

  - Bandwidth

  - Waveform repetition frequency

The request will be in the form of an http GET with the URL

http://<ip address:port>/rpei/status/<antenna>/<channel>

The table below gives the status returns

| Parameter – V5 | Type | Note |
| --- | --- | --- |
| version | long | header version |
| processing Chain | long | 1 if processing chain is running, 0 otherwise |
| transferring data | long | 1 if the system is currently processing data |
| number of receivers | long | The number of receivers this antenna is currently configured to use |
| master receiver | long | The index for the master receiver. This receiver is used for timing and must be actively transferring data in order for a data transfer to succeed |
| active receivers | long[number of receivers] | One value for each receiver. The value is 1 if the receiver is transferring data, and 0 otherwise. |
| Data type | long | 0 for radar data, 1 for sounder data |
| frequency | float | The waveform frequency in MHz. If transferringData is 0, this value is undefined |
| bandwidth | float | The frequency bandwidth in KHz. If transferingData is 0, this value is undefined. |
| wrf | float | The waveform repetition frequency in Hz If transferingData is 0, this value is undefined. |

For the transfer, the client will use google protocol buffer. In terms of the protocol buffer, the transfer uses the following protocol definition in the appendix.

# Http Error Codes

If a call reaches the server but then fails, the server will return an appropriate status message. The return status values used by the web server are:

- 400 – the request is not a valid rpei request

- 404 – the request refers to a type of rpei request which does not exist

- 520 – a transfer request cannot be serviced because the Cap Demo3 real-time system is not running

- 525 – a transfer request cannot be serviced because the Cap Demo3 real-time system is not taking data. In the case of server being run in test mode, this status return means that the RDM test file is not present or not readable.

- 530 – a transfer cannot take place because the Cap Demo3 real-time system is not processing data on the specified channel

- 540 – a transfer cannot take place because the client has asked for sounder data on a channel which is processing Radar data or has asked for radar data on a channel which is processing sounder data.

# Requests for Streaming RDM data

## Protocol

The requests will go from the Exmore system to the RPEI processing system using an http connection. In order to allow for easy conversion between the byte orders of the two systems, the parameter data for the start Transfer request and response will be serialized using google protocol buffers.

When the RPEI acquisition component on the Exmore system wants to start transferring RDM data from the CD3 system it starts by sending a request to process the desired channel of the desired antenna. It then sends requests to initiate transfer for each desired data stream. Each of these requests will return the header fields for the stream. At that point the acquisition component can create a disk files and can write the headers to the data base. For each stream, the acquisition then asks for each CPI of the data successively. As each line comes back, the acquisition component writes it to the file. Transfers can be halted by either side of the link. If the acquisition component wishes to start a different file, it sends a halt request (Request to transfer – Halt). If the CD3 side needs to terminate the request it will respond to a data request with a response indicating that -1 lines were transferred.

The initial implementation of this interface will only support one channel at a time. If the server receives a request to initiate a transfer while a request is active, the second request will be rejected.  If the server goes more than 1 minute without a request it will assume that the transfer is over. Subsequent deployments of this system will support the ability to support multiple concurrent channels by using multiple computers.

## Request to Process a channel – Start

This request tells the RPEI processor to process data from a specific channel of a specific receiver. The request contains the CPI parameters for the operator stream, plus the BCC parameters for this data acquisition. The request returns a process Id which is used by the subsequent commands to identify the processing chain set up by this request.

This request will be in the form of an http POST, with the URL	http://Ip_address/rpei/processChannel/<antenna>/<channel>

The request will contain the following mandatory parameters as detailed below

| Description | Range/Possible Values | Required Format | Comments |
| --- | --- | --- | --- |
| Version | 2 | short | Version of the request |
| Sweeps per CPI | 64, 128, 256, 512 | short | The number of sweeps per spectral line for the operator stream |
| SLI | Coherent integration time / power of 2 | float | time interval between spectral lines for the operator stream |
| SAP | 0, 1 | int | If 1, use Spatial adaptive processing |
| Rdm Norm | 0, 1 | Int | If 1, use rdm normalization |

The following additional parameters specifying the waveform must also be specified by this request (Note – this additional parameters are taken from reference 1

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Date/Time Stamp |  | double | seconds since Jan-1-1970 (Unix time_t) |
| MOC ID | 1,2,3,4 |  |  |
| Operating Frequency | greater than 0 | float | frequency in MHz |
| Sweep Rate | 100,200,400,500,1000,2000,2500,4000 | float | sweep rate in kHz/sec |
| Swept Bandwidth | 4,8,10,20,40,80,100 | float | bandwidth in kHz |
| Transmitter Number | 1 to 6 | long | transmitter |
| Transmit requested power | 0 to 500 | float | requested power from transmitter in kW |
| Transmit Antenna | ALF, BVO, CHR, DLT, GLF, 2HT, HT1, HT2, 2IN, IN1, IN2, NAC | char[3] | antenna id (Alpha, Bravo, Charlie, Delta, Golf, Hotel (single, in1, in2), India (single, in1, in2), none |
| Transmit Antenna Slew | +/- XX.X | degrees | direction relative to boresight. Valid values depend on antenna |
| Transmit Antenna vertical slew | “H”, “L”, “N” |  | H and L valid only for Hotel and India. |

The return value will be a process Id, which is used by subsequent requests to identify the processing chain set up by this request.

| Parameter – V5 | Type | Note |
| --- | --- | --- |
| process id | long |  |

## Request to Process a channel – Stop

This request tells the RPEI processor to stop processing the radar data and halts all data streams associated with the processing chain.

This request will be in the form of an http GET, with the URL

http://Ip_address/rpei/haltProcessing? Id=processingIdValue

Where processingIdValue is the value returned by the processChannel request.

## Request to Initiate Radar Data Transfer – Start

This request initiates transfer for a specific stream. The request contains the type of stream, and a specifier for when it should start. In the case of the operator stream, the request also contains additional data specifying the azimuth and beamwidth. The return will contain the header information for the stream, and an identification key to be used when asking for additional lines.

This request will be in the form of an http POST, with the URL

http://Ip_address/rpei/startTransfer? Id=processingIdValue

Where processingIdValue is the value returned by the processChannel request.

The request will contain the following mandatory parameters as detailed below

Field Description: The following input parameters must be specified with this request:

| Description | Range/Possible Values | Required Format | Comments |
| --- | --- | --- | --- |
| Version | Greater than 0 | short | Version of the request |
| stream type | 0 = operator stream 1 = narrow beam stream 2 = wide beam stream | short |  |
| Receive Beamwidth | 1.00 to 30.00 | float | Receive beamwidth in degrees (ignored unless type=0) |
| Receive Azimuth | 0.00 to 359.99 | double | Receive azimuth in degrees (ignored unless type=0) |
| Receive Elevation Angle | 0.00 to 45.00 | double | Receive vertical elevation angle in degrees. (ignored unless type=0) |
| Center Range | 0.0 to 12000.0 | float | Center processing range in km |
| Starting Time | -1 to 180 | float | Starting time for the data in number of minutes before the present time. A value of -1 means use the earliest time available. |

Response:	Header, Identification Key (see below)

On receiving the data, the legacy system will update its database and create a new disk file (ROP).

For the transfer, the client will use google protocol buffer. The google protobuf specification for the transfer is given in the appendix.

The startTransfer request will return a header for the RDM data for the stream.

### Header Data

Each header returned will consist of the standard ROP header for the narrow-beam return followed by the following additional information:

- The number of sweeps per CPI. This is a 16-bit integer.

- The CPI overlap percentage. Identification Key of the initiated transfer process

char – 8 bits, short – 16 bits, long – 32 bits, float – 32 bits, double – 64 bits

The table below gives the values as they will be encoded within the RDM file. All values will have the byte order consistent with that used by Sun Sparc.

| Parameter – V5 | Type | Note |
| --- | --- | --- |
| SFFT version | long | 3 for RDM Version 1 4 for RDM created from RASPS data (not currently supported through this interface) 7 for latest version |
| SFFT ID | char[15] | Format: SFFTYYYYMMDDNNN where NNN is a daily SFFT sequence number |
| Num Sweeps | long |  |
| Controls Obj Exists | long | Should always = 1 |
| Controls Obj Index | long | Should always = -1 |
| Controls version | long |  |
| Range Weighting | long | 0=False, 1=True |
| ROP Params Obj Exists | long | Should always = 1 |
| ROP Params Obj Index | long | Should always = -1 |
| ROP version | long | 7 for latest version 6 for rdm files created before August 2017 |
| Raw File Name Length | long |  |
| Raw File Name | char | Repeat Based on Raw File Name Length |
| File Size | long | Size of Raw File |
| RF Collection Input Source | long | 0=BFN_A_B, 1=Humline, 2=Buckoff, 3=External, 4=Off |
| Band | long | 0=None, 1=Low, 2=Mid, 3=High, 4=Off |
| Baseline | float | Meters |
| Delay Bias | float | Meters |
| Elevation Angle | double | Degrees |
| Steer Angle | double | Degrees |
| Freq | float | MHz |
| Cal Freq | float | MHz |
| OP Mode | long | 1=FMUP, 2=FMDOWN, 3=CW |
| Sweep Rate | long | KHz/sec |
| Sweep Delay | float | Msec |
| Chan A Atten | long | dB |
| Chan B Atten | long | dB |
| Video Amp | long | 0=False, 1=True |
| Video Filter | long | 0=False, 1=True |
| Sync Check | long | 0=False, 1=True |
| Swept Bandwidth | long | KHz |
| AOI Length | long |  |
| AOI | char | Repeat Based on AOI Length |
| Start Time (sec) | ulong | Seconds |
| Start Time (msec) | short | msec |
| Stop Time (sec) | ulong | Seconds |
| Stop Time (msec) | short | msec |
| Data Type | long | 0=Radar/Sync Check, 1=Noise Cal, 2=Amp Cal, 3=Humline, 4=Dynamic MDS, 5=CW Tone, 6=Sweeping MDS, 7=None, 8=Unknown |
| PRF | float | Hz |
| Center Recorded Range | float | Km This is the range that is associated with the sweep delay and may not be the center of the recorded range. It is the center range displayed. |
| Max Recorded Range | float | Km (Set to UAR) |
| Min Recorded Range | float | Km (Set to 0) |
| Max Unambig Range | long | Km |
| Pts per Sweep | short | PRF 100Hz: 250, PRF 50Hz: 500, PRF 25Hz: 1000, PRF 12.5Hz: 2000 |
| Antenna Type | long | 0=Flex, 1=Fixed, 3=Unknown |
| Num Range Bins | short |  |
| Injection Level | long | dBm |
| Mechanical Boresight | double | Degrees |
| Receive Azimuth | double | Degrees (mechanical boresight + relative steer) for the narrow-beam return. |
| Tilt Angle | double | Degrees |
| Element Spacing | float | Meters |
| Baseline Elements | long | Baseline in elements (i.e. Baseline / Element Spacing). This is the separation of phase centers |
| ARI Factor | float | ARI/Receiver Beamwidth for the narrow beam return. |
| ARI | float | Degrees |
| Bev Type | long | 0=Unknown, 1=A, 2=B, 3=C, 4=D |
| Beam Element | long | Elements per beam – Identifies processing mode: 112 – RDM Version 1 with SFFT Processing Mode 113 – RDM Version 1 NOT SFFT Processing Mode |
| Receiver Beamwidth | float | Degrees |
| Tx Power | long | Kw |
| Tx Chain ID | long | 90=Chain 1, 91=Chain 2, 92, Chain 3, 107=Chain 4, 121 = Chain 5 |
| Tx Antenna ID | long |  |
| Tx Beam Azimuth | double | Degrees |
| Tx ID | long | 93=Tx 1, 94=Tx 2, 95=Tx 3, 96=Tx 4, 97=Tx 5, 98=Tx 6 |
| Exciter Freq | float | MHz |
| Exciter Sweep Rate | long | KHz/sec |
| Exciter Swept BW | long | KHz |
| Exciter Op Mode | long | 1=FMUP, 2=FMDOWN, 4=CW |
| MGC Gain | float | dB |
| Sidelobe Level | long | dB |
| BFN ID | long | 0=None, 1=A1, 2=B1, 3=B2, 4=B3, 5=C1, 6=C2, 7=B4, 8=B5, 9=C3, 10=D1, 11=D2 |
| Receiver ID | long | 37=Rcvr 1, 38=Rcvr 2, 39=Rcvr 3, 40=Rcvr 4 |
| Sounder ID | long | 30=Sndr 1, 31=Sndr 2, 32=Sndr 3, 33=Sndr 4, 123 = Sndr 5, 124 = Sndr 6 |
| Subclutter Visibility | float | dB |
| Local Noise | float | dBm |
| Peak Power | float | dBm |
| Collection type | Int | 0=None, 1=Unknown, 2=OTH, 3=LOS, 4=Mixed Initially OTH for freq<30 MHz, LOS for freq>30 MHz |
| Sweeps Per CPI | short | Number of sweeps per CPI CIT (seconds) = Sweeps Per CPI / PRF |
| CPIOverlap | float | CPI overlap percentage This is equal to 1 – (desired SLI / (Sweeps Per CPI / PRF)) or 1 – (SLI / CIT). This value is multiplied by 100 to form a percentage |
| SLI | float | Spectral Line Interval - time between successive CPI’s in seconds |
| NumberOfCPIs | short | The total number of CPIs if known, otherwise 0 |
| SamplesPerCPI | long | The number of complex values in each CPI |
| Identification Key | short | Identified the transfer process initiated |
| Version 7 or higher |  |  |
| rpei Flag | long | Always 1 |
| channelId | long | Receiver Channel id used for generating raw data |
| Sap Flag | long | 1 if Spatial Adaptive Processing was used, 0 otherwise |
| rdmNormalization Flag | long | 1 if rdm normalization was used, 0 otherwise |
| CIT | float | CPI integration time in seconds |

The data transfer between the client and the network server will encode these values as a google protocol buffer.  The google protobuf definition for the header is in the appendix.

## Request for Radar Data

This request gets the next set of intensity-azimuth data from the RPE Prototype. The request will return the next CPI

This command takes the following form:

GET: 	http://Ip_address/rpei/transferData? Id=identificationKeyValue

Where:	IdentificationKeyValue is the identifier returned by the command to start the transfer

Response:	the number of CPI’s returned, followed by IQ Data (see below)

### RDM Radar Data

The RDM data consists of a set of complex values. Each complex value will consist of a real part followed by an imaginary part, both of which are in ieee-be single precision format. The complex values are interpreted like the complex values produced by current SFFT  processing after the A and B channels have been Doppler processed and combined. The amplitude of each complex value is proportional to the amplitude of the radar signal while the phase of each value is equal to the phase different between the two channels.

The data will be ordered as follows

- A block representing all the datacube values for a specific CPI

  - Within that block, a block representing all the datacube values for a specific range bin

    - Within that block a list of pairs representing all the Doppler values for that range bin. The list will be ordered so that the Doppler value for the highest frequency comes first. Each value is a complex number specifying the  radar signal in a given range/Doppler bin.

| Parameter | Type | Note |
| --- | --- | --- |
| Num CPI’s Returned | long | The number of spectral lines returned. It is set to -1 if a parameter has changed and a new ROP file needs to be created |
| Number of Values | long | The number of complex values being returned |
| Spectral Data | complex | Array[Num CPI’s returned*Num Range Bins* numDopplerBins] , with Doppler varying most rapidly. Two 32-bit floats (ie one complex number) per value. |

## Request to Initiate Transfer - Halt

This is a request to halt a previously initiated transfer.  It takes the following form:

GET:  http://Ip_address/rpei/haltTransfer? Id=identificationKey

Where:

IdentificationKey is the identifier returned by the command to start the transfer

For the operator stream, it is possible to send a halt transfer message, followed by a startTransfer message at a different azimuth or beamwidth. Note that this second startTransfer message can take advantage of the startingTime parameter to take a second look at data which has already been received.

# Offline RPEI Processing

In the offline process, the Cap demo system converts a set of raw Cap demo data to an RDM file. The client will send a parameter file over the network to a location the RDM offline process has access to, and then use ssh to invoke the offline RDM process over the network.  The command to start the offline process will have the form

rdmProcess parameterFile

Where parameterFile is the name of the parameter file. The parameter file will be an xml file with the following parameters:

- Path for the raw data file. The path must be on a disk mounted by the rpei processing system.
The current implementation of the software assumes the following convention for the directory structure:

  - A base path, which can be anything

    - The date, in the form YYYY-MM-DD (e.g 2017-05-04)

      - Channel number in the form chanNB1, chanNB2, etc

        - The individual files for that date and channel

Under this convention separate antenna arrays are indicate by changes in the base path.

- An optional parameter indicating how many minutes of data need to be processed. If the parameter is absent, the data in the specified file will be processed.  If the number of minutes is greater than the amount of data in the file and a file with the additional time is in the same directory, that file will also be opened and processed.

- Azimuth

- Beamwidth

- Elevation

- Center recorded range

- Sweeps per CPI

- SLI in seconds. For backward compatibility it is also possible to specify CPI overlap percentage in place of SLI

- SAP flag (optional, the default value is 0, which means no SAP)

- RDM normalization flag (optional, the default value is 0, which means no RDM normalization)

- Name for the output Rdm file

The parameter file has the following form:

<?xml version="1.0" encoding="UTF-8"?>

<rdmParameters>

<RawFilePath value=”path for the raw input file” >

<duration value=”number of minutes”/>

</RawFilePath>

<RpeiInputParameters>

<parameter name="azimuth" value=azimuth in degrees/>

<parameter name="beamwidth" value=beam width in degrees />

<parameter name="elevation" value=elevation in degrees />

<parameter name="centerRange" value=center range in kM/>

<parameter name="rangeWeighting" value="1" />

<parameter name="sweepsPerCPI" value=sweeps per CPI />

<parameter name="sli" value=sli in seconds />

<!—optional parameters -->

<parameter name=”SAP” value=SAP flag />

<parameter name=”rdmNorm” value=RDM normalization flag />

</RpeiInputParameters>

<rdmFilePath value="path for the output rdm file" />

</rdmParameters>

# Sounder Data requests

Sounder data is taken on a regular schedule. At any given time, each antenna array is collecting a quasi-vertical sounder, collecting a wide angle backscatter sounder (called oblique sounders on Exmore/Scimitar), or waiting for the next collection to start.  A schedule file on the system indicates exactly when each sounder collection starts and which receivers it uses. The quasi-vertical sounders are received by one receiver element and are not beam-formed. The wsbi sounders are received by several elements and are beam-formed into a small number of beams referred to as “sectors”.  The number of beams to be produced by the beamforming operation and their values will be defined in a configuration file on the system. In addition to the beam values specified in the configuration file, the RPEI system will produce an additional beam whose azimuth is specified by the operator.

At the present time the Scimitar/Exmore software cannot handle more than one azimuth of sounder data. Therefore for the present time, the ability to compute other azimuths will not be implemented. The quasi-vertical sounding data will not be collected using the antennas/receivers used by rpei.

The oblique sounders are collected from one of two transmitting antennas. The rpei software will determine which transmitting antenna to use by looking at the requested azimuth value. The same transmitting antenna will be used for all azimuths returned by a given response.

The number of frequencies, frequency increment, number of delays and delay increment are set in a configuration file. For the first release the values are set to be compatible with Scimitar:

- Frequencies go from 8 MHz and ending with 35 MHz in 20 kHz steps, for a total of 1350 frequencies.

- Time delays should correspond to ranges of 0 to 12000 km for a total of 1600 delays. (range = delay/speed of light)

There are two methods of transferring sounder data – Online transfer and offline transfer. For online transfer the client will ask for a data stream containing data from the sounder being collected currently. For offline processing, the user can convert raw sounder data from a set of files into a form which can be read in by Scimitar.

## Sounder scheduling

Sounder transmissions happen on a fixed schedule, with the time between successive sounders being 5 minutes. The current implementation uses a schedule file to indicate when soundings are scheduled to start. If the client asks for a sounder more than 1 minute before the next sounding is scheduled to start the request will return an error. Otherwise the first call will block until the scheduled time, and then return with the header.

## Online Sounder Processing

In online processing the following workflow will occur:

- The client will ask for the current oblique sounder.

- The client will receive a header for the current sounder. If no oblique sounder is currently being collected the client will receive an http error code of 525.

- The client will make or more requests for more data. The return will contain

  - The number of frequencies being returned. In the case where all frequencies have been returned, this value will be -1

  - The number of sectors being returned. In the case of a quasi-vertical sounder this will be 1

  - The power in decibels of the sounder data.

The header format is similar to the format used by Scimitar.

### Initiate Sounder Data Transfer

This request will be in the form of an http POST , with the URL

http://Ip_address/rpei/initiateSounderTransfer/<antenna>/<channel>

The following input parameters must be specified with this request.

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| version | 1,2,3,… |  | version number |
| additional azimuth | -180 to 180 | degrees | an beam value that the system should return in addition to the configuraed values |

The response to this request will be a header containing the following information

| Parameter | Type | Note |
| --- | --- | --- |
| ION Version | Int | Should be 5 |
| Current Frequency | Double | MHz |
| Start Frequency | Double | MHz |
| Stop Frequency | Double | MHz |
| Ion Type | Int | 0=VI, 1=BOI, 2=FOI |
| Curr Time Length | Int | Length of Curr time string |
| Curr Time | Char | Scimitar format: JJJ-yy HH:mm:ss.zzz |
| Start Time Length | Int | Length of start time string |
| Start Time | Char | Scimitar format: JJJ-yy HH:mm:ss.zzz |
| Stop Time Length | Int | Length of Stop time string |
| Stop Time | Char | Scimitar format: JJJ-yy HH:mm:ss.zzz |
| In Progress Flag | Int | 0=False, 1=True |
| Sweep Delay | Float | Msec |
| AOI Length | Int |  |
| AOI | Char | Repeat Based on AOI Length |
| Format Type | Int | 0=50 bit, 1=16 bit, 2=32 bit (Nov 2013 format), 3=32 bit Extended (March 2014 format) |
| Sweep State | Int | 0=Start, 1=Middle, 2=End |
| Operating Freq | Double | MHz |
| Tx Antenna Length | Int |  |
| Tx Antenna | Char | Repeat Based on Tx Antenna Length |
| Color threshold | Int | Not filled in |
| Array boresight | Float |  |
| Beamwidth | Float | Receiver beamwith at the average frequency (degrees) |
| Number of Azimuths | Int | Number of azimuths being sent |
| Azimuths | Float | List of azimuths being sent |
| Total scans | Int | The number of scans (frequency values) being sent |
| Number of delays | Int | The number of delay values being sent |
| Start delay | Float | Initial delay value in seconds |
| End delay | Float | Final delay in seconds |
| Identificaton Key | long | a temporary key used to identify the session for subsequent transfers. This will not be part of the OI file. |

For an online request, the header response is encoded as a google protocol buffer. For the offline request (see the section “Offline Sounder Processing”) the header is written to a file as big-endian binary data.

### Transfer sounder data

This request gets the nest set of frequencies for the current sounder.

The request takes the following form:

GET:	 http://Ip_address/rpei/transferSounderData? Id=IdentificationKey

Where:

IdentificationKey is the identification key returned by the previous initiate sounder data transfer request

The returned data is encoded as big-endian values. The returned data will consists of one float32 value per sounder value, representing the sounder power in decibels. The data will be ordered as follows:

A block representing a single frequency

Within that block, a set of blocks representing each sector (qvi data has one sector)

The sounder values in order of increasing delay

| Parameter | Type | Note |
| --- | --- | --- |
| Number of frequencies | long | The number of frequencies being returned in this request. If there is no more data it is -1 |
| Number of sectors | long | The number of sectors returned at each frequency |
| Number of Values | long | The number of sounder values being returned. |
| Sounder data | float32 | The power values in dB for the sounder |

### Halt Sounder Data Transfer

This is a request to halt a previously requested transfer. It takes the following form

GET:	 http://Ip_address/rpei/haltSounderTransfer? Id=IdentificationKey

Where:

IdentificationKey is the identification key returned by the previous initiate sounder data transfer request

## Offline Sounder Processing

In the offline sounder process, the Rpei system converts a set of raw Cap demo data to data file having the following format:

- A file header. This contains the same information as the online header. The values are all in big-endian byte order.

- Sounder data. The data is in Big-endian order. It consists of the data for the first frequency for all ranges, followed by the data for the next frequency, and so on.

.The client will send a parameter file over the network to a location the offline sounder process has access to, and then use ssh to invoke the offline sounder process over the network.  The command to start the offline process will have the form

processOfflineSounder parameterFile

Where parameterFile is the name of the parameter file. The parameter file will be an xml file with the following parameters:

- The desired azimuth

- The time for the sounder. You specify this in terms of hour (UTC) and minute. The seconds input should be -1 – in normal operation the rpei software will determine the seconds entry from the sounder schedule.

- The path for the input raw files. This should be the path that contains all the sounder data for that date and channel.

- The full path for the output file.

The parameter file has the following form:

<?xml version="1.0" encoding="UTF-8"?>

<sounderParameters>

<SounderInputParameters>

<parameter name="azimuth" value=azimuth in degrees />

<parameter name="hour" value=UTC hour value for the sounder collection time />

<parameter name="minute" value=minute after the hour for the sounder />

<parameter name="second" value="-1" />

</SounderInputParameters>

<ionoFilePath value=full path for the output file/>

<sounderRootPath value=full path for the channel directory of the input files/>

</sounderParameters>

# Request Clutter Map

The clutter map request transfers a kmz file containing a clutter map. The request must be made to an antenna and data channel which is currently processing RPEI data. It will return two outputs:

- A clutter map for the current time, and for the frequency and range of angles currently being processed.

- A target map consisting of the return averaged over azimuths as a function of ground range.

If the antenna and channel specified by the request are not currently processing data the request will return with an http error code of 530.

The inputs to the clutter map request are

- A table for converting radar range to ground range. The clutter map function will use this table as an interpolation table.

If the user does not specify a table the function will return a clutter map in which the range is equal to radar range rather than ground range. For that reason it is important for the client to specify some sort of registration table even if it is only based on a fixed-height mirror model. Ideally Exmore should use the same ionosphere registration that it uses for registering tracks.

This request will be in the form of an http POST, with the URL

http://Ip_address/rpei/clutterMap/<antenna>/<channel>

The request takes the following parameters:

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| version | positive | long | version number |
| mapType | 1,2,3 | long | 1=clutter map 2 =subclutter map |
| registration table size | greater than 2 | long | size of the registration table |
| radar range table | positive, monotonically increasing | float[registration table Size] | radar range values in km |
| ground range table | positive | float[registration table Size] | ground range values in km |

The return data will contain the following

| Parameter | Type | Note |
| --- | --- | --- |
| Number of kmz values | long | The number of bytes being returned for the kmz |
| clutter map | byte[Number of values] | The kmz data corresponding to the clutter map |

# Unimplemented features

The features described below will not be implemented in the initial delivery of the RPEI system.

## Sync Check

The sync check allows an operator to monitor proper synchronization between the transmitter and receiver antennas. RPEI could provide a sync check by monitoring the Doppler value of the ground return signal, which should be present at all cpi’s at the range bin representing a range of 0.

# Appendix Google Protobuf Definitions

Parameters and header information transmitted using the http requests will be encoded using Google Protocol buffers. The following definition specify the google protocol buffer classes which wil be used.

## Status return

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "rpeiStatus";

message Status {

optional	int32	version			=	1	[default = 1];

optional	int32	processingChain =	2;	// The processing chain is running

optional	int32	transferringData = 	3;	// The system is transferring data

optional	int32	numberOfReceivers = 	4;				//total number of receivers

optional	int32	masterReceiverIndex = 	5;				//index of the master receiver

repeated 	int32	activeReceiver 		= 	6;	//0 if receiver is inactive, 1 if transferring data

optional	float	frequency 		=	7	[default = 0];	//The central frequency in MHz

optional	float	bandwidth 		=	8	[default = 0];	//The frequency bandwidth in KHz

optional	float	wrf 			=	9	[default = 0];	//The repetition frequency in Hz

}

## Start Transfer Request

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "startTransfer";

message StartTransfer {

//user input parameters

optional	int32	version		=	1	[default = 1];

optional	int32	streamType		=	2;	//0 = operator, 1 = narrow beam, 2 = wide beam

optional	float	beamwidth 		=	3;	// ignored if streamType != 0

optional	double	azimuth	=	4;	// ignored if streamType != 0

optional	double	elevation	=	5;	// ignored if streamType != 0

optional	float	centerRange	=	6;	// ignored if streamType != 0

optional	float	StartingTime	=	7	[default = -1.0];

}

## RDM Header

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "ropHeader";

message RopHeader {

optional	int32	SFFT_version	=	1	[default = 1];	//	SFFT version

optional	string	SFFT_ID	=	2	;	//	SFFT ID

optional	int32	Num_Sweeps		=	3	;	//	Num Sweeps

optional	int32	control_obj_exists	=	4	[default = 1];	//	Controls Obj Exists

optional	int32	Control_obj_index	=	5	[default = -1];	//	Controls Obj Index

optional	int32	Control_version	=	6	[default = 1];	//	Controls version

optional	int32	range_weighting	=	7	[default = 1];	//	Range Weighting

optional	int32	ROP_obj_exists	=	8	[default = 1];	//	ROP Params Obj Exists

optional	int32	ROP_obj_index	=	9	[default = -1];	//	ROP Params Obj Index

optional	int32	ROP_version	=	10	[default = 5];	//	ROP version

optional	int32	 raw_file_str_len	=	11	[default = 0];	//	Raw File Name Length

optional	string	 raw_filename	=	12	[default = ""];	//	Raw File Name

optional	int32	raw_file_size	=	13	[default = 0];	//	File Size

optional	int32	collection_input	=	14	;	//	RF Collection Input Source

optional	int32	band			=	15	;	//	Band

optional	float	baseline		=	16	;	//	Baseline

optional	float	delay_bias		=	17	;	//	Delay Bias

optional	double elevation_angle	=	18	;	//	Elevation Angle

optional	double steer_angle	=	19	;	//	Steer Angle

optional	float	freq			=	20	;	//	Freq

optional	float	cal_freq		=	21	;	//	Cal Freq

optional	int32	op_mode		=	22	;	//	OP Mode

optional	int32	sweep_rate		=	23	;	//	Sweep Rate

optional	float	sweep_delay	=	24	;	//	Sweep Delay

optional	int32	chan_a_atten	=	25	;	//	Chan A Atten

optional	int32	chan_b_atten	=	26	;	//	Chan B Atten

optional	int32	video_amp		=	27	;	//	Video Amp

optional	int32	video_filter	=	28	;	//	Video Filter

optional	int32	sync_check_flag	=	29	;	//	Sync Check

optional	int32	swept_bw		=	30	;	//	Swept Bandwidth

optional	int32	AOI_str_len	=	31	;	//	AOI Length

optional	string	aoi		=	32	;	//	AOI

optional	int64	start_time_sec	=	33	;	//	Start Time (sec)

optional	int32	start_time_msec	=	34	;	//	Start Time (msec)

optional	int64	stop_time_sec	=	35	;	//	Stop Time (sec)

optional	int32	stop_time_msec	=	36	;	//	Stop Time (msec)

optional	int32	data_type		=	37	;	//	Data Type

optional	float	prf				=	38	;	//	PRF

optional	float	center_recorded_range	=	39	;	//	Center Recorded Range

optional	float	max_recorded_range	=	40	;	//	Max Recorded Range

optional	float	min_recorded_range	=	41	;	//	Min Recorded Range

optional	int32	max_unamb_range	=	42	;	//	Max Unambig Range

optional	int32	pts_per_sweep	=	43	;	//	Pts per Sweep

optional	int32	antenna_type	=	44	;	//	Antenna Type

optional	int32	num_range_bins	=	45	;	//	Num Range Bins

optional	int32	injection_level	=	46	;	//	Injection Level

optional	double	mechanical_boresight	=	47	;	//	Mechanical Boresight

optional	double	receive_azimuth	=	48	;	//	Receive Azimuth

optional	double	tilt_angle		=	49	;	//	Tilt Angle

optional	float	element_spacing	=	50	;	//	Element Spacing

optional	int32	baseline_ele	=	51	;	//	Baseline Elements

optional	float	ari_factor		=	52	;	//	ARI Factor

optional	float	ari				=	53	;	//	ARI

optional	int32	bev_type		=	54	;	//	Bev Type

optional	int32	beam_element	=	55	;	//	Beam Element

optional	float	rcvr_beamwidth	=	56	;	//	Receiver Beamwidth

optional	int32	tx_power1		=	57	;	//	Tx Power

optional	int32	tx_chain_id	=	58	;	//	Tx Chain ID

optional	int32	tx_antenna_id	=	59	;	//	Tx Antenna ID

optional	double	tx_beam_azimuth	=	60	;	//	Tx Beam Azimuth

optional	int32	tx_id1		=	61	;	//	Tx ID

optional	float	exciter_freq	=	62	;	//	Exciter Freq

optional	int32	exciter_sweep_rate	=	63	;	//	Exciter Sweep Rate

optional	int32	exciter_swept_bw	=	64	;	//	Exciter Swept BW

optional	int32	exciter_op_mode	=	65	;	//	Exciter Op Mode

optional	float	mgc_gain		=	66	;	//	MGC Gain

optional	int32	sidelobe_level	=	67	;	//	Sidelobe Level

optional	int32	bfn_id		=	68	;	//	BFN ID

optional	int32	receiver_id	=	69	;	//	Receiver ID

optional	int32	sounder_id		=	70	;	//	Sounder ID

optional	float	subclutter_vis	=	71	;	//	Subclutter Visibility

optional	float	local_noise	=	72	;	//	Local Noise

optional	float	peak_power		=	73	;	//	Peak Power

optional	int32	sweeps_per_cpi	=	74	;	//	Sweeps Per CPI

optional	float	cpi_overlap	=	75	;	//	CPIOverlap

optional	float	sli	           =   76  ;   //  Spectral line interval (sec)

optional    int32 number_cpis   =   77  ;   //  Total number of cpi's

optional    int32 samples_per_cpi =   78  ;   // samples per cpi

optional	int32	identification_key	=	79	;	//	Identification Key

optional	int32 rpeiFlag			=	81	[default=1];	//always 1

optional	int32 channelId			=	82	[default=1];	//channel which did original collection

optional	int32 sapFlag			=	83	[default=0];	// 1 if sap was used in processing

optional	int32 rdmNormalizationFlag	=	84	[default=0];	// 1 if rdmNormalization was used

optional	float CIT			=	85;	// integration time

}

## ClutterMap request

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "clutterMapRequest";

message Request {

optional	int32	version	=	1	[default = 1];

optional	int32	mapType 	=	2;		// 1=clutter, 2=subclutter

//used to convert the map to ground range

optional	int32	registrationTableSize = 	4	[default = 1];	//number of values in registration table

repeated 	float	radarRangeTable 	= 	5;	//radar range values in km

repeated 	float	groundRangeTable 	= 	6;			//radar range values in km

}

## Sounder Request

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "sounderRequest";

message SounderRequest {

optional	int32	version		=	1	[default = 1];

optional	float	azimuth 	=	2;		// desired azimuth for OI sounder

}

## Sounder Response

package rpeiInput;

option java_package = "edu.mit.ll.rpei";

option java_outer_classname = "sounderResponse";

message SounderResponse {

optional	int32		ionversion		=	1	[default = 5];

optional	double		currentFrequency 	=	2;				// MHz. use min frequency

optional	double		startFrequency		=	3;				// MHz

optional	double		stopFrequency		=	4;				// MHz

optional	int32 		ionType			=	5	[default=1];		// 0: VI; 1: OI; 2: FOI

optional	string	currTime		=	6;				// scimitar format JJJ-yy HH:mm:ss.zzz

optional	string	startTime		=	7;				// scimitar format JJJ-yy HH:mm:ss.zzz

optional	string	stopTime		=	8;				// scimitar format JJJ-yy HH:mm:ss.zzz

optional	int32		inprogressFlag		=	9	[default=1];

optional	float		sweepDelay		=	10;				//in msec

optional	string	aoi			=	11	[default=""];

optional	int32		formatType		=	12	[default=3];		//0=50 bit, 1=16 bit; 2=32 bit; 3=32 bit extended

optional	int32		sweepState		=	13	[default=0];		//0=start, 1=middle; 3=end

optional	double	operatingFrequency	=	14;				//MHz - use avg frequency

optional	string	txAntenna		=	15;				//antenna name

optional	int32		colorThreshold		=	16	[default=0];		//not used

optional	float		arrayBoresight		=	17;

optional	float		beamWidth		=	18;				//beamwidth at agv frequency in degrees

optional	int32		numberOfAzimuths	=	19;				//number of azimuths

repeated	float		azimuths		=	20;				//list of azimuths

optional	int32		totalScans		=	21;				//number of scans (frequencies)

optional	int32		numberOfDelays		=	22;

optional	float		startDelay		=	23;

optional	float		endDelay		=	24;

optional	int32		identificationKey	=	25;		//unique id

}