| # Interface Control Document # For the SRC # RCS-5 and Ancillary Equipment Obsolescence Replacement |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contract No: |  |  |  |  |  |  |  |  |
| SDRL Sequence No: |  |  |  |  |  |  |  |  |
| Prepared for: BAE Systems Information & Electronic Systems Integration Prepared by: BAE SYSTEMS Australia Limited ACN 008 423 005 Taranaki Road, Edinburgh Parks, Edinburgh SA 5111 |  |  |  |  |  |  |  |
|  | Name |  | Position |  | Signature |
| Prepared by: | Frank Hali |  | Senior Software Engineer |  |  |
| Approved by: | Robert Averay |  | Senior Design Engineer |  |  |
|  | Mike Hopton |  | System Technical Authority |  |  |
| Authorised by: | Laurie Burgess |  | Project Manager |  |  |

# DOCUMENT CHANGE HISTORY

| Issue | Change Number | Description of Change | Date of Issue |
| --- | --- | --- | --- |
| 1 |  | Initial version |  |
| 2 |  | GD Review Amendments | 16-Aug-2011 |
| 3 |  | Amendments from GD Discussion | 19-Aug-2011 |
| 4 |  | Amendments from GD Proposal Letter / BAE Amendments | 27-Aug-2012 |
| 4 | B | BAE Amendments | 30-Nov-2012 |
| 4 | C | CDR Amendments | 30 Jan 2013 |
| 4 | D | GD Amendments | 30 Apr 2013 |
| 4 | E | GD Amendments (Option 1B) | 20 Jun 2013 |

# INTERFACE REVISION CHANGE HISTORY

The interface version describes the layout of messages specified in this document and is provided to ensure all parties are supporting the required revision of the interface. It is always a positive integer. The interface version is provided at runtime via several message formats to allow runtime checking (ref: 3.1.7 and 3.3.1).

If for any reason the format of any message or data is changed in such a way as to cause misinterpretation by any user of this interface then the interface version number must be incremented,

| Issue | Document Issue | Description of Change | SRS Version |
| --- | --- | --- | --- |
| 1 | 4B | Implement interface version numbers | 30 Nov 2012 |
| 2 | 4C | Modified OVI tasking command | SRC V1.30 |
| 3 | 4D | Change sounder start to include DRx decimation | SRC V1.31 |
| 4 | 4E | Change sounder start command and data format | SRC V1.32 |

# CONTENTS

1	Scope	5

1.1	Identification	5

1.2	System Overview	5

1.3	Document overview	5

1.4	Applicable Documents	5

1.5	Definitions	5

2	Time Protocol Interface	7

2.1	Description	7

2.2	Interface	7

2.2.1	IP Address	7

2.2.2	Command Messages	7

3	Sounder Receive System Interface	8

3.1	Introduction	8

3.1.1	System Overview	8

3.1.2	System Initialisation	8

3.1.3	Receiver Pooling	9

3.1.4	Receiver Availability	9

3.1.5	Command Processing & Timing Constraints	10

3.1.6	TCP/IP Sockets	11

3.1.7	Determining the IP Address of the SRS	11

3.1.8	SRS Data Interface	13

3.2	SRS Interface	15

3.2.1	Description	15

3.2.2	SRS Message Format	15

3.2.3	SRS Command Messages	16

3.2.4	SRS Response Messages	18

3.3	SRS System level commands	20

3.3.1	Connecting to the SRS	20

3.3.2	Report Error Code Description	21

3.3.3	Report System Status	22

3.3.4	Watchdog Timer	24

3.4	SRS Sub-System Commands	25

3.4.1	Historical Clear Channel Monitor Sub-System	25

3.4.2	Real-Time Clear Channel Monitor Sub-System	28

3.4.3	Ionospheric Sounder Sub-System	34

ANNEX A - SRS Interface Handshaking Example	38

ANNEX B - SRS Warning Messages	39

ANNEX C - SRS Error Messages	40

FIGURES

Figure 1 – Sounder Receiver System	8

Figure 2 – SRS Initialisation Sequence	9

Figure 3 – SRS Start Command Processing (simplified)	10

# TABLES

Table 1: Terminate Data message	13

Table 2: Message Delimiters	15

Table 3: Sub-System Codes	16

Table 4: Command Codes	17

Table 5: Data Response Message Parameters	19

Table 6: Connect ID Definition	20

Table 7: Status Report Header	22

Table 8: Status Report Block	22

Table 9: Status Report Message Codes	23

Table 10: Timer Message Header	24

Table 11: HCCM Start command parameters	25

Table 12: HCCM Halt command parameters	26

Table 13: HCCM Data Header	27

Table 14: HCCM Spectral Data	27

Table 15: RTCCM Start command parameters	29

Table 16: RTCCM Halt command parameters	29

Table 17: RTCCM Marker command parameters	30

Table 18: RTCCM FFL fixed parameters	32

Table 19: RTCCM FFL parameters	32

Table 20: RTCCM Data fields	33

Table 21: OVI Task Parameter Block Header	34

Table 22: OVI Task Slot Parameters	35

Table 23: OVI Halt command parameters	36

Table 24: OVI Sweep (Sounding) Header	37

Table 25: OVI Scan Data Block Header	37

Table 26: SRS Warning Messages	39

Table 27: SRS Error Messages	40

# Scope

## Identification

This document applies to "RCS-5 and Ancillary Equipment Obsolescence Replacement”. Specifically, this document is the Software Design Description for the Sounder Receiver System.

## System Overview

The Host is responsible for the logistical support of the sounder receive and timing systems. The equipment at the Receive (Rx) site was installed more than 20 years ago and was considered to be ‘state of the art’ at that time.

BAE SYSTEMS Australia has already delivered an upgrade to the existing Tx sounder sub-system and a Performance Monitoring and Diagnostics sub-system to monitor the sounder purity of the Tx sounder sub-system.  BAE SYSTEMS Australia has also delivered an upgrade to the existing Tx timing system.

This project will provide replacement of the RCS-5 Sounder Receivers, the Real Time Clear Channel Monitors (RTCCM), the Historical Clear Channel Monitor (HCCM) and the Real Time Monitors (RTM).  The replacement will also provide control and diagnostic software, Sounder Receive System (SRS), for the monitoring of the new sounder’s performance.

In addition to the replacement of the RCS-5 Sounder hardware this project will also upgrade the Receiver site timing system to an equivalent standard as the Transmit site.

## Document overview

This document is Software Interface Control Document for the interface between the Receiver and Sounder Control Systems and the BAE SYSTEMS Australia's Sounder Subsystems.

## Applicable Documents

- GD – OT Workstation / CEC Sounder ICD, 17 August 2010.

- GD Request for Proposal, 12 October, 2011

## Definitions

BCC	Beam steer control Computer

BFN	Beam Forming Network

BW	Bandwidth

RTCCM	Real Time Clear Channel Monitor

DOS	Digital Receiver Operating System

FFL	Forbidden Frequency List

HCCM	Historical Clear Channel Monitor

HF	High Frequency

Hz	Hertz

IP	Internet Protocol

kHz	Kilohertz

MHz	Megahertz

NTP	Network Time Protocol

OIS	Oblique Incidence Sounder

OVI	Oblique and/or Vertical Incidence Sounders

RMS	Receiver Monitor Scope

SRS	Sounder Receive System

Scan	A collection of samples with a coherent integration time

Sweep	A coherent collection of scans for the specified frequency range (Sounding)

TCP	Transmission Control Protocol

VIS	Vertical Incidence Sounder

# Time Protocol Interface

## Description

The timing system replacement uses two XLi Time and Frequency Units, as primary and secondary units.  Each unit is fitted with the Network Time Server Option 87-8017.

Time information will be served out to the Exmore and R&D Servers via Ethernet.

## Interface

Network Time Protocol (NTP) V3/V4 (RFC1305) will be used by default, to serve time to the Exmore and R&D Servers over Ethernet.

### IP Address

Each NTP server will have a default IP address.  This IP address shall be reserved to ensure that it will not cause an address contention and also that it cannot be inadvertently commanded.

The IP address of the primary NTP server installed in the system shall be (192.168.1.117)

The IP address of the secondary NTP server installed in the system shall be (192.168.1.118)

### Command Messages

Request messages to the NTP server for time shall be IAW that specified by the NTP V3/V4 protocol.

# Sounder Receive System Interface

## Introduction

### System Overview

The Sounder Receive System (SRS) is a software suite composed of several CSCIs and provides the command interface to the SRS Client (Exmore, Test Harness, etc.) for the SRS tasking. The SRS is configured on a redundant computer environment in which all computers will run the complete, identical SRS software suite. When the SRS Client connects to one of the available SRS suites the computer system hosting that suite will become known as the primary computer. The SRS only supports a single primary computer at one time.

The SRS is shown diagrammatically below:

Sounder

Receiver Control

Sounder Receiver Display

SRS Client

History Data

Operator GUI

DRx Operating System

RX1..RX6

Command

Control & Data

Data

Command

Data

Command

Figure 1 – Sounder Receiver System

### System Initialisation

When the SRS Client connects to an SRS, data and control paths must be established to each BAE SYSTEMS Advanced Digital Receiver (DRx). This is achieved by commanding the DRx Operation System (DOS) to configure the appropriate connections to the DRx and perform the required initialisation and configuration. This process will typically take several seconds per receiver and must be accommodated for within the SRS Client whenever a new connection is established to the SRS.

The flow diagram below depicts the SRS start-up summary:

Start DOS and SRD processes

Y

N

SRS Start Command?

Client Stop Command?

Y

N

Read Configuration File

Start SRS Cleint TCP/IP server thread

BEGin DOS sessions to each DRx and send response message to the SRS Client

Halt all tasking & reset receivers; close TCP/IP connections to DOS and SRS Client

Read DOS/DRx status messages and display

Read SRS Cleint command

Start

Figure 2 – SRS Initialisation Sequence

### Receiver Pooling

The SRS is offered to the SRS Client as a pool of DRx’s which can be used interchangeably between sounder sub-systems. Each receiver is identified to the SRS Client by a unique identification code (e.g. RX1 to RX6) which is mapped by the SRS at initialisation to a specific physical digital receiver.

Note that as the input to each receiver is controlled by the Exmore (via the video matrix switch) and the function of each receiver is controlled by the Exmore (via the SRS as described by this document) the Exmore is free to perform whichever sub-system tasking is required on whichever receiver is available at the time this tasking is required. However, Exmore must be aware that due to the conflicting nature of sub-system tasking some sub-systems cannot co-exist on a single receiver simultaneously.

### Receiver Availability

At program initialisation the SRS configures all receivers offered in the configuration file and makes these receivers available to the SRS Client for tasking. After SRS initialisation, the primary computer maintenance console will provide the operator with visual feedback on the status of each receiver and the current state of tasking.

In addition to this feedback, the operator console will provide the operator with the ability to take one or more receivers offline (i.e. remove receiver availability from the receiver pool) for the purpose of hardware resets, diagnostics, etc. When a receiver is taken offline by the operator, all tasking will be immediately aborted. Further, all SRS tasking commands issued while a receiver is offline will be returned with an error response message to indicate this condition.

### Command Processing & Timing Constraints

All commands received by SRS are validated for syntax and receiver availability and then responded to. As the SRS does not queue tasking (i.e. does not support submitting multiple commands asynchronously to run sometime in the future and expect SRS to schedule these chronologically), all new tasking (upon receipt) is scheduled according to the simplified flow chart below. Any tasking that SRS sends to DOS will cause an immediate channel halt on the requested receiver. Note that if a running task is interrupted the data output is undetermined (i.e. none, some or all of the tasking data may be sent over the data socket).

Due to the setup constraints, commands should arrive at DOS at least 1,000mS before the start epoch. Therefore, allowing for network and processing delays all commands should arrive at SRS 1,500mS before the start epoch to ensure correct task start time is achieved. Tasking that does not meet the setup constraints will be rejected by the SRS (command response code will indicate an error). The flow diagram below shows SRS command processing logic:

Parse Start command

HCCM Start?

Send command to DOS halting existing HCCM tasking (if any)

Rx in use for RTCCM or ovi?

Send Error Response

(Tasking conflict)

N

Y

N

Y

Continue

Send command to DOS halting existing RTCCM tasking (if any)

RTCCM Start?

Rx in use for HCCM?

Send Error Response

(Tasking conflict)

Y

N

Y

Reorder hourly schedule allowing existing OVI run (if any) to complete

OVI Start?

Rx in use for HCCM?

Send Error Response

(Tasking conflict)

Y

N

Y

N

N

Figure 3 – SRS Start Command Processing (simplified)

### TCP/IP Sockets

The SRS uses TCP/IP sockets for control and data transfer to and from the SRS Client system. The SRS implements a TCP/IP socket server listening for incoming connections on specific TCP/IP port numbers for control and data.

A single socket server is provided to interface to the SRS Client system over which all commands for all sub-systems will be passed. Only ASCII string messages are transmitted over this socket.

A single server socket servicing multiple client connections is provided for sub-system data. Each sub system will produce its data in accordance with the format and content specified by this ICD. All data read from data sockets will be in binary format.

### Determining the IP Address of the SRS

The SRS is a redundant computer configuration with a primary and a secondary computer. There are no functional differences between the primary and secondary computers; the only difference being their IP address. The primary computer is defined as the computer that the SRS Client system has established a TCP/IP connection to and is accepting SRS Commands. Typically, the rack mounted hardware console displays will be monitoring the primary SRS but these are operator selectable.

Note that although the computer systems and software suites are redundant there is no capability for automatic failover in the event of a failure of the primary system. If the primary computer fails the SRS Client will need to establish a connection to an alternate computer and re-initialise all tasking.

To connect to the SRS the SRS Client system will establish a TCP/IP client connection to the first node address on its list. If the connection is successful, then all subsequent communication will be to this (primary) computer. Note that if the SRS already has a connection established at the time of a new connection request, the SRS will reject the connection request. SRS active tasking will not be altered until a Halt command is received.

If the connection attempt to the first node on the list fails, the SRS Client system is to attempt to connect to the second node in the list, and so on. Connection attempts shall continue to loop through the list of SRS computers until a connection to the primary SRS is established.

Upon the successful establishment of a TCP/IP socket connection to the SRS, the SRS Client will send a connection request message which will be acknowledged by the SRS.

#### IP Numbers

Below are listed the IP numbers of the computers in the SRS system. The search order of this list must be in the order given below:

The IP address of the first SRS computer shall be: 192.168.1.123

The IP address of the second SRS computer shall be: 192.168.1.124

#### TCP/IP Command Port Number

Below is the TCP/IP port number that must be used for connecting to the SRS TCP/IP command server:

SRS Command Interface Port Number: 5561 (0x15B9)

#### TCP/IP Data Port Number

Below is the TCP/IP port number that must be used for connecting to the SRS TCP/IP data server:

SRS Data Interface Port Number: 53985 (0xD2E1)

### SRS Data Interface

When a command is successfully received by the SRS, and that command causes data to be generated, the SRS will reply over the command socket with a Data Response message detailing the connection details for the data transfer. To receive this data, a TCP/IP connection will need to be established with the data server.

#### Initiating Data Transfer from SRS

The SRS will respond to ‘Start’ commands that generate data with a Data Response Message (Ref: 3.2.4.2). The eight ASCII text character Connect ID included in this response message is used to establish a direct data path to the requested sub-System.

Establishing a data path is achieved by opening a TCP/IP client socket to the provided node IP and port address. Upon establishing a TCP/IP client connection the Connect ID must be sent across this connection. This establishes a client socket to the appropriate data source within the SRS.

Note that all commands to the data socket do NOT support command delimiter characters.

After receiving the Connect ID on the data socket the SRS will map the requestor to the specified receiver by channel number and data type as specified within the Connect ID.

Note that the Connect ID only changes when the receiver serial number, channel number or data type changes. Therefore, the SRS Client does not need to re-establish this connection with each Start command, thus gaining efficiencies on socket establishment overheads. This is determined by comparing any new Connect IDs with previous connections Connect IDs.

After a data source has been mapped using the Connect ID the data server will ignore all incoming data on the data socket with the exception of the Termination message.

Note that DOS can support up to six data connections for a given ConnectID. This means up to six independent TCP/IP socket connections can be made to receive the identical data from DOS. As these are TCP/IP sockets they may all originate from a single node or may come from multiple nodes on the network.

#### Reading Data from the SRS

All data sent by the SRS will be in a binary format, the contents being dependant on the sub-system generating the data. All data will be grouped in blocks with a header prefixing each block. Refer to the individual sub-systems for details of specific data block formats (ref: 3.4).

Note that Microsoft Windows sends all binary data in Little-Endian format.

#### Terminating Data Transfer from SRS

Upon completion of receiving all data for a given sub-system, the SRS Client can release the data path by transmitting the terminate message (shown below) and closing the socket. Upon receipt of the termination message the SRS will flush any outstanding binary data and formally close the TCP/IP socket. The format of the termination message follows:

| String | Description |
| --- | --- |
| CLOSE | A five character ASCII text message terminating the TCP/IP data connection. |

Table 1: Terminate Data message

Note that if the sub-system remains active the data path can be re-established at any time using the appropriate command response message on a new connection to the data server.

Note that SRS supports multiple data clients to the data collector allowing identical data to be transmitted to multiple clients if required.

## SRS Interface

### Description

All commands are sent to the SRS over the command socket interface and are in the form of ASCII text messages containing a variable number of parameters (dependant on the particular command).

The SRS command interface does not queue commands (i.e. all SRS Client commands are implemented immediately upon receipt and superseded by any subsequent commands to the sub-system), therefore, any queuing of commands to accommodate timing must be performed in the computer sending commands to the SRS Interface. The interface will acknowledge all commands with a response message.

Command message parameters are to be separated by a single space character (ASCII decimal 32) except for the first and last characters (immediately adjacent the message delimiters).

Numerical values include a decimal point, if needed. Leading and trailing zeros are not required, thus “8” = “08.00”. Where values may be +/-, positive values are assumed unless preceded by a minus sign; the plus sign is not used. Text values are case sensitive.

### SRS Message Format

All Command and Response messages to and from the SRS are formatted as follows:

- Start of message,

- System code,

- Command/Response code,

- Parameters …,

- End of message.

Note that all characters in the TCP/IP stream will be ignored until the valid ‘start of message’ character is received. No commands will be processed or executed until a matching ‘end of message’ character is received. All characters received after the ‘end of message’ character are to be ignored until the next ‘start of message’ character.

Message delimiter characters are shown in Table 2:

| Character | Description |
| --- | --- |
| [ | Start of message (ASCII decimal 91). |
| ] | End of message (ASCII decimal 93). |

Table 2: Message Delimiters

### SRS Command Messages

All Command messages are formatted according to the Command and Response Message Format outlined in section 3.2.2.

To command the SRS a valid SRS Command message must be received. The SRS will respond to all Command messages with a Response message. Refer to the sub-system commands for specific responses. Examples of SRS Command messages follow:

Example 1:	“[C S 123 AB 6]”

Where:	“C” is the sub-system Code for the RTCCM

“S” is the Start Command

“123 AB 6” are the sub-system specific Start Command parameters

This example will start the RTCCM sub-system passing “123”, “AB” and “6” as formal parameters.

Example 2:	“[S R]”

Where:	“S” is the System Code for the SRS

“R” is the Report Status command code

This example is commanding the SRS to provide the SRS Client with a Status Report.

#### Sub-System Codes

The sub-System code specifies what sub-System function the SRS is to implement. The valid SRS sub-System codes are shown in Table 3:

| Character | Description |
| --- | --- |
| C | Real-Time Clear Channel Monitor sub-system (RTCCM). |
| H | Historical Clear Channel Monitor sub-system (HCCM). |
| I | Ionospheric Sounders sub-system (OVI). |
| S | The SRS system. Used for SRS control and status messages. |

Table 3: Sub-System Codes

#### Command Codes

The Command code specifies the action to be performed on the specified sub-system. Following the Command code there may be a number of optional parameters, the number of which will be dependent on the sub-System and Command. The valid SRS Command codes are shown in Table 4:

| Character | Description |
| --- | --- |
| E | Request an Error or Warning code text description (SRS only) |
| S | Start sub-system command (all sub-systems). |
| H | Halt sub-system command (all sub-systems). |
| M | Marker data (RTCCM sub-system only). |
| F | Forbidden Frequency data (RTCCM sub-system only). |
| R | Report system status (SRS only) |
| T | Timer command (SRS only). |

Table 4: Command Codes

### SRS Response Messages

All Response messages are formatted according to the Command and Response Message Format outlined in section 3.2.2.

When a Command message is received, the SRS will reply over the command socket with an ASCII text response message. The contents of the response will depend on the originating command; however, every response message is prefixed by the sub-system and command code of the originating command; for example:

Command:	[H S …]	start a HCCM sub-system command sent to the SRS

Response:	[H S n …] 	response message from the SRS

Directly following the sub-system and command code in the response message will be the Error Code indicating the success of the command.

Following the Error Code will be a variable number of parameters or text depending on the specific command.

In its simplest form the response message will only contain the first three fields; e.g.

Example:	[C M 0]	RTCCM marker command executed successfully

#### Error Codes

A numeric status number is included in all response messages. A zero indicates that the previous command was successful. A negative integer indicates an error was detected and a positive integer indicates a warning condition exists. Refer Annex B and Annex C for a complete listing of all warning and error conditions.

A descriptive text message for any status code can be obtained by sending the SRS the ‘E’ command.

#### Data Response Message

A specific form of the response message known as the Data Response message is returned by any sub-system command that will cause data to be generated.

The Data Response message advises the SRS Client that data for a command will be made available by the SRS (specifically DOS) when connecting to the SRS data socket and providing the supplied connection code.

The Data response message also provides a unique sequence number. This sequence number will be embedded in the header of each data block to uniquely associate a data block with a task.

Syntax:	[S C n …]

Where:	“S”	indicates the sub-system of the originating command

“C”	indicates the command of the originating command (always a Start command)

“n”	is the error code

“…”	are the parameters returned for this command (ref: Table 5)

Field Description:	The following details will be included in the response message:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Node Name | V4 IP Address | n.n.n.n | The v4 IP address of the data source node. |
| Port Number | 1..65,535 | Unsigned 16bit integer | The destination node v4 IP port number in base 10 format. |
| Connect ID | a..Z, 0..9 | Eight ASCII characters | A connect message used to establish a unique data path to the SRS. |
| Sequence No | 1.. 232 | Unsigned 32bit integer | A unique task sequence number to identify all data produced for this task. |

Table 5: Data Response Message Parameters

Example:	[H S 0 192.168.1.123 53985 CT222000 123]

For this HCCM Start command response the error status was successful (zero) and all data generated by this task will be available on node 192.168.1.123:53985. The data socket connection identifier (Connect ID) is “CT222000” and the packet data will be identified in the data block header by the sequence number 123.

Note that Microsoft Windows sends all binary data in Little-Endian format.

#### Composition of the Connect ID

The Connect ID is an encoded field that describes the data connection to the data generation component of DOS. This allows various clients to correctly connect to the correct data stream. This field is encoded thus:

| Description | Range Values | Data definition | Comments |
| --- | --- | --- | --- |
| Client Type | C | Char constant | Instructs DOS that this is a data client. This will always be ‘C’ for SRS generated requests. |
| Data Type | A..Z | Char | ASCII character data type identifier allocated during the system design. This is used by DOS to associate the data generated to the data requestor as multiple data types may be available. |
| Rx Sn | 000..FFF | Char[3] (hex) | The actual serial number (as derived from the MAC number) of the receiver. |
| Ch No | 000..005 | Char[3] (decimal) | Receiver channel number. This will be determined by the SRS depending on the tasking requirements. |

Table 6: Connect ID Definition

## SRS System level commands

### Connecting to the SRS

#### SRS Client Initiating Control of the SRS

When the SRS Client initiates control of the SRS a Start command message must be sent. This should be the first command after establishing a TCP/IP command socket to the SRS (ref: 3.1.7).

Note that the Start command will cause the SRS to establish control and data paths to each DRx identified in the configuration file via DOS. Establishing these data paths and enumerating the receivers is a lengthy process so SRS will return a response message immediately the command is initiated. This allows the SRS Client to continue with other processing while waiting for the SRS to become available. To monitor the progress of the start-up, the SRS Client can poll the SRS using the Status Report command. This command details the success of each receiver’s initialisation (Ref: 3.1.5).

An SRS response message will be sent indicating the status of the Start command and the Interface version and node name of the SRS. Note that connecting to the SRS with the incorrect interface version number will cause a warning response message.

This command takes the following form:

Syntax:	[S C v n]

Where:	“S”	is the SRS sub-system code (must be ‘S’)

“C”	indicates the command which must be ‘S’

“v”	 will be the interface version number of the transmitting node

“n”	 will be the node name of the transmitting node

Example:	[S S 1 SrsClientNodeName]

Response:	[S S 0 1 SrsNodeName]

In this example the SrsClientNodeName computer has requested the SRS to ‘Start’ and is at interface version number 1.

A worked example is provided in ANNEX A - SRS Interface Handshaking Example.

#### SRS Client Terminating Control of the SRS

When the SRS Client terminates control of the SRS a termination message must be sent. This will cause the primary SRS to close the TCP/IP command socket, halt all tasking, reset all receivers and close all data paths to DOS. SRS will then continue monitoring the Command server socket for new control connections.

An SRS Response message will be sent before this command is implemented. The termination action will only be taken if the Response message indicates a normal (zero) status (i.e. the command is syntactically correct).

Example:	[S H]

Response:	[S H 0]

### Report Error Code Description

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

This command is used to obtain a descriptive text message for a given error code. Error codes are fully described in section 3.2.4.1, Annex B and Annex C.

This command takes the following form:

Syntax:	[S E n]

Where:	“S” indicates the sub-system code (which must be ‘S’ for the SRS)

“E”	indicates this is a Report Error Code Description command

“n”	is the error code for which a description is being requested.

Example:	[S E -2] sent to SRS.

[S E 0 Connection is already established] response from SRS.

Note that the response to the Report Error Code command reflects the status of the command. Therefore, if the command executed normally, the returned error code will be zero.

### Report System Status

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

When a Status Request Command is received, the SRS will reply over the command socket with a response message reporting the status of all receivers.

This command takes the following form:

Syntax:	[S R]

Where:	“S”	is the sub-system code (ref: Table 3)

“R”	indicates this is a Report Status command

Example:	[S R]

#### Status Report Response Message

The Status Request Response message takes the following form:

Syntax:	[S R n c …]

Where:	“n”	is the command error code to the [S R] command (ref: 3.2.4.1)

“c”	 is the status block count (ref: Table 7)

“…”	 are message details described (ref: Table 8)

Note the message response to this command has a fixed and variable component. The first four fields of the response are always present. The fourth field will describe the number of variable parameter blocks to follow.

Field Description:	The following details must be specified (in the order listed in Table 7).

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Status Blocks | 0..n | Integer | Specifies the number of fixed format status blocks in this message. Refer Table 8 for block format. |

Table 7: Status Report Header

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Rx ID | RX1..RXn | Text | Identifies Rx chain being reported. |
| Sub-System | Refer: Table 3 | Single ASCII character | Sub-System code to which the receiver is tasked. |
| Status | Refer: Table 9 | Single ASCII character | Device status |

Table 8: Status Report Block

Example:	[S R]	request SRS to send a Status message

Response:	[S R 0 7 RX1 C R RX1 I R RX2 S F RX3 H R RX4 S A RX5 S A RX6 S U]

In this example the command executed successfully (error code zero) and there are seven blocks of information in the status report:

- RX1 is running RTCCM tasking,

- RX1 is also running OVI tasking,

- RX2 is allocated to the SRS and has failed,

- RX3 is allocated to the HCCM sub-system and is running an HCCM task,

- RX4 is online and available but has no tasking,

- RX5 is online and available but has no tasking,

- RX6 offline and unavailable.

#### Status Request Message Status Codes

The status codes are used to indicate the state of a receiver and its availability to tasking.

The possible Status codes included in the Status Response Message are shown in Table 9:

| Character | Description |
| --- | --- |
| F | Receiver is online but tasking has Failed (always allocated to SRS sub-system) |
| A | Receiver is online and Available (always allocated to SRS sub-system) |
| R | Receiver is currently Running sub-system tasking (HCCM, RTCCM or OVI) |
| U | Receiver is offline and Unavailable (always allocated to SRS sub-system) |

Table 9: Status Report Message Codes

### Watchdog Timer

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The Watchdog Timer command is provided as a keep-alive timer to ensure the TCP/IP command socket connection has not been lost.

When a Watchdog Timer command is received, the SRS will reply over the command socket with an ASCII text response message which includes the user defined text sent in this message. This allows the sender to embed packet tracking, etc. should it be required.

This command takes the following form:

Syntax:	[S T …]

Where:	“S”	is the SRS sub-system code (must be ‘S’)

“T”	indicates this is a Timer command

“…”	 optional sending node text (ref: Table 10)

Field Description:	The following details may be specified

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| User defined | User defined | ASCII text | Provides optional user defined data which is to be returned in the response message. This allows the sender to maintain implementation specific message tracking if required. |

Table 10: Timer Message Header

Example:	[S T 321 Keep Alive]

Response:	[S T 0 321 Keep Alive] response from SRS

## SRS Sub-System Commands

### Historical Clear Channel Monitor Sub-System

The Historical Clear Channel Monitor (HCCM) sub-system produces a single scan of the requested data and then stops. The HCCM does not have the capacity to run continuously and therefore must be ‘Started’ for each collection.

The SRS will only accept HCCM commands to a receiver that is not tasked with any other sub-system tasking. That is, if a receiver has RTCCM or OVI tasking active when a HCCM task is submitted for that receiver the SRS will ignore the command and return an error response message. If the receiver is currently performing HCCM tasking when a new HCCM tasking command is received the existing task will be halted immediately and the new task implemented. Figure 3 refers.

Available commands are detailed below:

#### Historical Clear Channel Monitor Start Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The HCCM Start command is used to command the HCCM. The Start command will be responded to with a Data Response message (ref: 3.2.4.2).

This command takes the following form:

Syntax:	[H S …]

Where:	“H”	is the HCCM sub-system code

“S”	command code for the Start command

“…”	 mandatory parameters as detailed below

Field Description: 	The following parameters must be specified (in the order listed in Table 11).

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Rx No | RX1..RXn | RXn | Identifies receiver chain for collection. The ‘RX’ prefix is a required constant. |
| Start Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the start frequency of the task. |
| End Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the end frequency of the task. |

Table 11: HCCM Start command parameters

Note all HCCM tasking will use a resolution bandwidth specified in the SRC configuration file.

Example:	[H S RX3 21500000 26500000]

Response:	[H S 0 …] data response message (ref: 3.2.4.2 for details)

Start HCCM processing on Receiver number 3 centred at 24MHz with a frequency span of 5MHz.

#### Historical Clear Channel Monitor Halt Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The HCCM Halt command is used to command the HCCM to halt all data gathering immediately. Any task scheduled, but not yet started, will be purged. The Halt command will be responded to with a generic response message. A Halt command sent to a receiver that is not currently tasked for this sub-system will force a channel disable instruction to be sent to the receiver and then return a success response code to the SRS Client (i.e. Halting a non-existing task will not return an error or warning).

This command takes the following form:

Syntax:	[H1 H2 …]

Where:	“H1”	is the HCCM sub-system code

“H2”	command code for the Halt command

“…”	 mandatory parameters as detailed below

Field Description:	The following parameter must be specified:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Rx No | RX1..RXn | Text | Identifies appropriate Receiver |

Table 12: HCCM Halt command parameters

Example:	[H H RX3] Stop HCCM on Receiver 3 immediately.

Response:	[H H 0]

Note that the halt command will force the closure of the server Data Interface socket.

#### Historical Clear Channel Monitor Data Interface

HCCM data will become available upon completion of a task. Output from each task can be read from the data socket as a single block of data as described below:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Sync Block | A5A5A5A5 | 32bit Unsigned Integer | Packet alignment word. |
| Task Seq No | 1.. 232 | 32bit Unsigned Integer | A unique sequence number to identify all data produced for this task. |
| Sounding Type | H | 32bit Unsigned integer | The task type specified by the Start command. The low byte represents ASCII value of the Task Type ASCII character. |
| Start Seconds | 1-86,400 | 32bit Unsigned integer | The start time of this task in seconds since 2400 UTC. |
| Start Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The starting frequency in Hz. |
| End Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The ending frequency in Hz. |
| Data bytes | 1.. 231 | 32bit Signed Integer | The number of bytes that must be read from this data stream (i.e. excluding this header) before the next block header. |

Table 13: HCCM Data Header

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Spectral Power | ±0..32,768 | 16bit Signed Integer | A scaled value in dBm * 100 (e.g. 123 represents 1.23dBm). The data stream will contain Data Bytes/2 samples per block |

Table 14: HCCM Spectral Data

Note that Microsoft Windows sends all binary data in Little-Endian format.

### Real-Time Clear Channel Monitor Sub-System

The Real-Time Clear Channel Monitor (RTCCM) sub-system produces a continuous stream of the requested data once it is ‘Started’, until a ‘Halt’ command is received. If a new ‘Start’ command is given to a running RTCCM the existing tasking will be halted immediately and the new tasking implemented.

The SRS will only accept RTCCM commands to a receiver that is exclusively tasked with RTCCM tasking or shared with OVI tasking. That is, if a receiver has HCCM tasking active when a RTCCM task is submitted for that receiver the SRS will ignore the command and return an error response message. Figure 3 refers.

Note that the RTCCM data, although available, is not typically used by Exmore and is currently only used by the BAE SYSTEMS RTCCM operator displays (SRD).

Note that due to the fact that the data stream can be restarted mid-stream the Data Sequence number will always be set to 0xA5A5A5A5 to allow readers to correctly realign.

Available commands are detailed below:

#### Real-Time Clear Channel Monitor Start Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The RTCCM Start command is used to command the RTCCM to start gathering data immediately. The Start command will be responded to with a Data Response message. The command syntax follows:

This command takes the following form:

Syntax:	[C S …]

Where:	“C”	is the RTCCM sub-system code

“S”	command code for the Start command

“…”	 mandatory parameters as detailed below

Field Description: 	The following parameters must be specified in the order listed below.

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Rx No | RX1..RXn | “RX”n | Identifies receiver chain for collection. The ‘RX’ prefix is a required constant. Only one receiver can be specified in the command. |
| WS No | WS1..WSn | “WS”n | ID of operator workstation(s) to display the RTCCM data. The ‘WS’ prefix is a required constant. Any number of workstations can be specified up to the maximum allowed in the SRC configuration file. |
| Start Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the start frequency of the task. |
| End Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the end frequency of the task. |

Table 15: RTCCM Start command parameters

Note all RTCCM tasking will dynamically adjust the resolution bandwidth to display the configuration file specified plot resolution (CCM_BARS).

Note the maximum frequency span that can be specified is 4.375MHz.

Note that a change in frequency (and not span) will cause the SRS to leave existing tasking to continue and only change the frequency of the receiver (i.e. the data stream will be continuous).

Note that at least one workstation must be specified with the maximum number of workstations limited to that specified in the SrcConfig.ini file. As workstations require RT-CCM data to be delivered from DOS there is a DOS imposed maximum of six only simultaneous data feeds limiting the number of simultaneous workstations to this number.

Note that if a workstation is allocated to different receiver without halting the original receiver and it is the last workstation for the original receiver a halt will be issued to the original receiver.

Example:	[C S RX1 WS3 WS5 20000000 20100000]

Response:	[C S 0 …] data response message (ref: 3.2.4.2 for details)

Start RTCCM processing on receiver number 1 at 20.05MHz with a bandwidth of 100 kHz; send output to Operator Workstations 3 and 5.

#### Real-Time Clear Channel Monitor Halt Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The RTCCM Halt command is used to command the RTCCM to halt all data gathering immediately. Any task scheduled, but not yet started, will be purged. The Halt command will be responded to with a generic response message.

This command takes the following form:

Syntax:	[C H …]

Where:	“C”	is the RTCCM sub-system code

“H”	command code for the Halt command

“…”	 mandatory parameters as detailed below

Field Description:	The following parameter must be specified:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Rx No | RX1..RXn | RXn | Identifies appropriate Receiver |

Table 16: RTCCM Halt command parameters

Example:	[C H RX3] Stop RTCCM on Receiver 3 immediately.

Response:	[C H 0]

Note that the halt command will force the closure of the server Data Interface socket.

#### Real-Time Clear Channel Monitor Marker Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The RTCCM Marker command is used to command the RTCCM to display a pair of markers on the specified operator workstation, indicating the current operating bandwidth and noise threshold.

The Marker command information will be stored with the workstation parameters and can therefore be sent at any time (i.e. workstation does not need to be tasked). The specified markers will be displayed on the workstation when the markers fall within the tasking frequency and power ranges.

Note that markers are not validated against tasking parameters and will only be displayed when they are within the tasking parameters.

Note that Marker parameters are immediately forwarded to the workstation for display.

This command takes the following form:

Syntax:	[C M …]

Where:	“C”	is the RTCCM sub-system code

“M”	command code for the Marker command

“…”	 mandatory parameters as detailed below

Field Description:	The following parameter must be specified:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| WS No | WS1..WSn | “WS”n | ID of operator workstation(s) to display the RTCCM data. The ‘WS’ prefix is a required constant. Any number of workstations can be specified up to the maximum allowed in the SRC configuration file. |
| Frequency 1 | 1,000,000..45,000,000 | Unsigned Integer (Hz) | Draw marker at this frequency on the RTCCM graphic display. |
| Frequency 2 | 1,000,000..45,000,000 | Unsigned Integer (Hz) | Draw marker at this frequency on the RTCCM graphic display. |
| Noise Threshold | ±0..32,768 | 16bit Signed Integer | Draw noise threshold marker on graphical display. A scaled value in dBm * 100 (e.g. 123 represents 1.23dBm) |

Table 17: RTCCM Marker command parameters

Note that at least one workstation must be specified with the maximum number of workstations is limited to that specified in the SrcConfig.ini file.

Example:	[C M WS2 WS3 1350000 13600000 -11350]

Response:	[C M 0]

Display a colour band from 13.5MHz to 13.6MHz and a horizontal marker at
-113.50dBm on workstations 2 and 3.

#### Real-Time Clear Channel Monitor Forbidden Frequency List

##### Description

The Forbidden Frequency List (FFL) is utilised by the RTCCM and is used to highlight forbidden frequencies on the frequency occupancy display (SRD). It is provided as a text data TCP/IP stream on an as-required basis by the SRS Client system. As the FFL is associated with a workstation ID it may be sent at any time. The SRS will cache the list for use with any current or future tasking.

##### Forbidden Frequency List Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The SRS accepts a list of individual start and stop frequency pairs to be used in the FFL for an operator workstation RTCCM display. There is no limit to the number of frequency pairs that can be listed in a single command message. This command will be responded to with a Command Response message.

This command can be sent at any time and will be immediately transferred and implemented on the specified operator workstation. The dynamic FFL will be stored and applied to all current and future tasking of the specified workstation.

A new FFL will overwrite the existing FFL (should one exist).

This command takes the following form:

Syntax:	[C F …]

Where:	“C”	is the RTCCM sub-system code

“F”	command code for the FFL command

“W”	workstation ID (ref: Table 18)

“…”	 repeating mandatory parameters as detailed below

Field Description:	The following parameter only appears once in this command:

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| WS Count | 1..max workstations | Unsigned Integer | The number of workstation IDs to follow. |
| WS No | WS1..WSn | “WS”n | ID of operator workstation(s) to display the RTCCM data. The ‘WS’ prefix is a required constant. Any number of workstations can be specified up to the maximum allowed in the SRC configuration file. |

Table 18: RTCCM FFL fixed parameters

Field Description: 	The following parameter pairs may be repeated as often as required:

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Start Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | The starting (included) forbidden frequency. |
| End Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | The ending (included) forbidden frequency. |

Table 19: RTCCM FFL parameters

Note that the FFL is sent immediately to the workstation for display.

Example:	[C F 1 WS1 WS3 20100000 20105000 15000000 15100000]

Response:	[C F 0]

Add a 20.100MHz to 20.105MHz frequency (5 kHz span) and a 15MHz to 15.1MHz frequency (100 kHz span) to the forbidden frequency list for workstation operators 1 and 3 RTCCM display.

##### Remove Forbidden Frequency List Command

Individual frequencies cannot be removed from any operator workstation FFL. When a FFL is sent to the operator workstation the existing FFL will be overwritten with the new FFL. Sending an empty FFL is used to remove an existing dynamic FFL.

Example:	[C F 1 WS1] Remove the dynamic FFL for operator workstation 1.

#### Real-Time Clear Channel Monitor Data Interface

RTCCM data will become available upon implementation of a Start command. Data can be read (should a client require it) from the data socket as a repeating stream of data as described below:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Sync Block | A5A5A5A5 | 32bit Unsigned Integer | Packet alignment word. |
| Task Seq No | 1.. 232 | 32bit Unsigned Integer | A unique sequence number to identify all data produced for this task. |
| Sounding Type | C | 32bit Unsigned integer | The task type specified by the Start command. The low byte represents ASCII value of the Task Type ASCII character. |
| Start Seconds | 1-86,400 | 32bit Unsigned integer | The start time of this task in seconds since 2400 UTC. |
| Start Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The starting frequency in Hz. |
| End Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The ending frequency in Hz. |
| Data bytes | 1..232 | 32bit Unsigned Integer | The number of bytes that must be read from this data stream (i.e. excluding the four words of this header) before the next block header. |
| Spectral Power | ±0..32,768 | 16bit Signed Integer | A scaled value in dBm * 100 (e.g. 123 represents 1.23dBm). The data stream will contain Data Bytes/2 samples per block |

Table 20: RTCCM Data fields

Note that Microsoft Windows sends all binary data in Little-Endian format.

### Ionospheric Sounder Sub-System

The Oblique and Vertical Incidence Ionospheric Sounder (OVI) sub-system accepts schedules for a sixty minute repeating cycle. The existing schedule will repeat every 60 minutes unless otherwise notified.

Available commands are detailed below.

#### Ionospheric Sounder Start Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The OVI accepts Start commands and will respond with a data response message. Upon implementation of the command, data will be transmitted to the data socket for collection.

An OVI Start message will contain all tasks (i.e. sounder sweeps) for the hourly schedule. The new schedule will be implemented immediately upon completion of any currently running sweep and will replace the existing schedule. The schedule will repeat continuously until another Start message with a new schedule is received or the schedule is halted using the Halt command.

Note that support is provided for SRS testing in an environment where sounders are started at set intervals (e.g. every eight minutes) rather than an absolute schedule. To support this environment a single sounder task may be entered with a negative slot count representing the number of seconds between Starts.

This command takes the following form:

Syntax:	[I S H …]

Where:	“I”	is the OVI sub-system code

“S”	command code for the Start command

“H”	is the header component of the command (ref: Table 21)

“…”	 variable number of parameters slots (ref: Table 22)

Field Description:	The following parameters must be specified once at the start of the command:

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Rx ID | RX1..RXn | “RX”n | Identifies Rx chain for collection. |
| Number of Slots | 0..99 | Unsigned Integer | The number of tasks in an hour. Each slot will contain the appropriate number of data items. If the number of slots is set to zero, all existing tasks will be purged; however, any running task will be allowed to complete normally. |

Table 21: OVI Task Parameter Block Header

Field Description:	The following parameters must be repeated for each slot:

| Description | Range/Possible Values | Required Format/ Unit | Comments |
| --- | --- | --- | --- |
| Sounding Type | F, B or V | Unsigned 8bit Integer (char) | Identifies the sounding type (Forward, Backward or Vertical) |
| Direction | N or S | Unsigned 8bit Integer (char) | Identifies the sounding direction (North or South). Note: For VI soundings this will be set to ‘N’ |
| Start Time | MM 00..59 SS 00..59.999999 | MM:SS.SSSSSS (seconds in µS resolution) | The tasks start time after the hour. Required range delay is to be added to the start time. |
| Start Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the start frequency of the OVI task. |
| End Frequency | 1,000,000..45,000,000 | Unsigned Integer (Hz) | This is the end frequency of the OVI task. |

Table 22: OVI Task Slot Parameters

Example:	[I S RX1, 12, V, N, 00:30.005, 1000000, 16000000, V, S, 5:30, 1000000, 16000000, V, S, 10:30, 1000000, 16000000, V, N, 15:30, 1000000, 16000000, V, N, 20:30, 1000000, 16000000, V, N, 25:30, 1000000, 16000000, V, N, 30:30, 1000000, 16000000, V, 35:30, 1000000, 16000000, V, N, 40:30, 1000000, 16000000, V, N, 45:30, 1000000, 16000000, V, N, 50:30, 1000000, 16000000, V, S, 55:30, 1000000, 16000000]

Note that commas or spaces (or both) can be used to delimit parameters.

Response:	[I S 0 …] data response message (ref: 3.2.4.2 for details)

This example shows a sounder equipment chain. Start sounder processing on receiver 1 (RX1) every 5 minutes starting 30 seconds after the hour. Each sounder task (sweep) is from 1MHz to 16MHz at a sweep rate of 100 kHz/second. For an Oblique sounding (F or B) 20,480 real samples will be returned every second; for a vertical sounding (V) 1,024 real samples will be returned every second. In this example, the first sweep is scheduled to start at 30 seconds past the hour with a 5mS range delay. The sounder schedule for this receiver will continue until a new Start or Halt command is received.

Alternate Start command format (BAE internal use) Example:

[I S RX1, -840, F, S, 00:30.005, 1000000, 16000000, 100000, 1000]

This example shows a sounder tasking that will start at 00:30.005 and reschedule itself every 840 seconds. This format supports sweep rate and Rx decimation as the last two parameters of the command. This alternate form of the Start command only supports a single (recurring) task and is provided for BAE internal use only.

#### Ionospheric Sounder Halt Command

This command must be formatted according to the Command and Response Message Format outlined in section 3.2.2.

The OVI Halt command is used to command the OVI to cancel the hourly schedule. The currently executing task (if any) will be aborted immediately. All future tasks will be purged. The Halt command differs from a Start command with zero slots in that it will immediately abort the currently running task while the zero slots Start command will allow the running task to complete normally. The Halt command will be responded to with a generic response message.

The Halt command is required to remove the Receiver specific child plot window on the Performance Display (SRR.exe).

This command takes the following form:

Syntax:	[I H …]

Where:	“I”	is the OVI sub-system code

“H”	command code for the Halt command

“…”	 mandatory parameters as detailed below

Field Description:	The following details must be specified:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Rx No | RX1..RXn | “RX”n | Identifies Receiver on which to Halt sounder tasking. A receiver without tasking will not generate an error to this command |

Table 23: OVI Halt command parameters

Example:	[I H RX3] Stop OVI gathering on Receiver 3 immediately.

Response:	[I H 0]

Note that the halt command will force the closure of the server Data Interface socket.

#### Ionospheric Sounder Data Interface

Ionospheric Sounder data will become available upon implementation of a Start command. Data can be read from the data socket as a repeating stream of data as described in Table 24:

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Sync Block | A5A5A5A5 | 32bit Unsigned Integer | Sweep packet alignment word. |
| Task Seq No | 1.. 232 | 32bit Unsigned Integer | A unique sequence number to identify all data produced for this task. |
| Sounding Type | F, B or V | 32bit Unsigned integer | The task type specified by the Start command. The low byte represents ASCII value of the Task Type ASCII character. |
| Start Seconds | 1-86,400 | 32bit Unsigned integer | The start time of this task in seconds since 2400 UTC. |
| Start Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The starting frequency in Hz. |
| End Frequency | 1,000,000..45,000,000 | 32bit Unsigned Integer (Hz) | The ending frequency in Hz. |
| Total Scans | 1.. 232 | 32bit Unsigned Integer | The total number of scans (and therefore Scan Block headers described in Table 25) to be sent for this sounding. |

Table 24: OVI Sweep (Sounding) Header

Note that the data will be written by the SRS to the data socket as it is generated. This will allow data to be delivered throughout the sweep (sounding) rather than a single burst at the end of the sweep (sounding). Typically, the data packet size will be determined by the receiver sample rate divided by two; therefore, a scan data block will be available every 500msecs.

| Description | Range/Possible Values | Required Format/Unit | Comments |
| --- | --- | --- | --- |
| Scan Seq No | 1.. 232 | 32bit Unsigned Integer | A unique sequence number to identify the current scan. Scan Sequence starts at 1 for the first scan and increments sequentially for subsequent scans. |
| Data bytes | 1.. 232 | 32bit Unsigned Integer | The number of bytes that are included in this scan. |
| Samples | ±8.4^-37 to 3.4^38 | 32bit IEEE floating point | Sounder sample as a single Real. |

Table 25: OVI Scan Data Block Header

Note that Microsoft Windows sends all binary data in Little-Endian format.

# ANNEX A - SRS Interface Handshaking Example

Below is a step-by-step example of commanding the SRS to perform an HCCM data collection:

- Determine the SRS host computer IP address (3.1.7)

- Open a client TCP/IP socket to SRS on the command interface port and send the start  command (ref: 3.1.7 & 3.3.1)
- send: “[S S 3 SrsClientNodeName]”
- recv: “[S S 0 3 SRS01]”

- Send the HCCM Start command message over the socket (ref: 3.4.1.1) 
- send: “[H S RX3 21300000 5000000]”
- recv: “[H S 0 192.168.1.123 53985 013CD123 1]”

- Open a client TCP/IP socket to ”192.168.1.123:53985” (ref: 3.1.8).
Note we are opening a specific data socket for a specific Connect ID. Each receiver/channel/system combination will open a unique socket path.
Note that the SRS can serve multiple data streams for a specific Connect IDs, simultaneously.

Send Connect ID parameter extracted from the command response message over the data socket (ref: 3.1.8.1 and 3.2.4.3):
- send: “CD013000”

- Continuously read binary data from data interface socket (ref: 3.1.8.2). The first four bytes are two 16bit unsigned integers describing the data to follow.
Note that if a new Start command is issued the task will be halted and restarted with the new parameters. This change will be indicated in the data stream by the Sequence Number in the block header.

- Send a keep alive timer message (ref: 3.3.4):
- send: “[S T 12434 TCP Keep Alive]”
- recv: “[S T 0 12434 TCP Keep Alive]”

- Send new HCCM tasking to SRS (ref: 3.4.1.1): 
- send: “[H S RX3 20300000 6000000]”
- recv: “[H S 0 192.168.1.123 53985 013CD123 357]”

- Continue reading data; note that the data socket (i.e. command response message) for a given Connect ID will always be the same allowing pre-opening of all data sockets if desired or simply opening once with the first Start command and continually reading binary data while new commands are issued over the command socket.

- Halt the HCCM System (ref: 3.4.1.2): 
- send: “[H H RX3]”
- recv: “[H H 0]”

- End a data stream from a specific receiver for a given System (ref: 3.1.8.3):
- send “CLOSE” over data socket allowing a delay to ensure delivery
- close TCP/IP data socket

- End a control session of the SRS (ref: 3.3.1.2):
- send: “[S H]”
- recv: “[S H 0]”
- close TCP/IP command socket

# ANNEX B - SRS Warning Messages

| Code | Message |
| --- | --- |
| 1 | Incorrect Interface version |

Table 26: SRS Warning Messages

# ANNEX C - SRS Error Messages

| Code | Message |
| --- | --- |
| -1 | Description of invalid error code requested |
| -2 | Client is already connected to SRS |
| -3 | Client has not yet Started the SRS |
| -4 | Invalid Receiver Identification |
| -5 | Error in tasking command |
| -6 | Receiver is offline and not available |
| -7 | Invalid Sub-System Code |
| -8 | Invalid Command Code |
| -9 | Message has incorrect number of parameters |
| -10 | Incompatible Tasking |
| -11 | Invalid Workstation Identification |
| -12 | Start time has elapsed |

Table 27: SRS Error Messages